| How It Was Done! |  | Car owner’s name Michael J Kurtzweil |
| --- | --- | --- |
| Year and model of car 1977 MGB |  | Town/city and state New London, WI |
| Who built or modified this car? I did |  | E-mail address Rotomike@charter.net |
| Engine (make, model, configuration, displacement, etc.) 1999 4.0 Land Rover From salvage yard |  | Engine electrical (including ignition) Delco electronic Ignition D&D Fabrications |
| Intake (manifold & carburetor, or fuel-injection specifics) Dual port Offenhauser manifold, Edlebrock carb D&D Fabrications |  | Cam Crower Cam- intake 258 duration, 430 lift |
| Cooling system Water pump and timing cover from D&D Fabrications MGB V8 radiator |  | Exhaust system Hugger headers-D&D Fabrications Single exhaust- Hushpower muffler |
| Transmission (type, make/model, ratio of top gear, etc.) 1987 Borg Warner T5 - .63 5th gear D&D Fabrications |  | Clutch System / driveshaft McCleod hydraulic clutch Modified S10 drive shaft- from D&D Fabrications |
| Rear axle (make/model, ratio, limited-slip?) Narrowed S10 rear end- 3.07 ratio. D&D Fabrications |  | Brakes Stock disk brakes in front, drum in rear Parking brake cable system from D&D Fabrications |
| Front suspension / steering Lowered 1” coil springs, Spax tube shocks |  | Rear suspension Stock leaf springs-1” lowering blocks Spax tube shocks |
| Wheels/Tires Dunlop bolt on wire wheels |  | Weight ? |
| Instruments Smiths/Cobra gauges from the Gaugeguys.com |  | Electrical (battery, charging, lighting, audio, etc.) Complete V8 wiring harness from Advanced Auto Wire |
| Date completed / Miles (or races) driven since completion June 13, 2009 1,200 miles |  | (racecars only) What class does this car compete in? |
| Other chassis modifications Hood Gas props from Little British Car. Steady Torque Bar from Clive Wheatley. Caster shims from front end from A&C Tucker Replaced all bushings with polyurethane. |  |  |
| Other body and interior modifications |  |  |
| Performance data It’s fast. Gear ratio lets me cruise highway at about 1800 RPM at 72 MPH. |  |  |
| Other important info (unusual features / historical notes / comments / tips / problems encountered during or after completion) I’d researched this project on and off for about 6 months. Then this winter I was going to pull the 4 cylinder to have the engine bay painted. One thing led to another and the decision was made to go with the V8. I found the engine in a salvage yard and broke it down in my shop at home. I relied heavily on Mark at D&D Fabrications for technical support and parts. I started in late December of ’08 and finished up in mid June, I was a little over a month behind of my schedule but things happen, lots of nights and every weekend for 6.5 months. I worked on the engine at home during the week and on the car on the weekends the first couple of months, when the engine was ready I took it to the club and then all my spare time and then some was spent at the club shop. I belong to the Fox Cities British Car Club and they have a great facility for working on your car. I couldn’t and wouldn’t have done it without the club. My profession is graphics so this was my first venture in this sort of project but I always wanted to see if I could do something like this. It was worth it but I wouldn’t do it again. Problems- the hydraulic clutch gave me some fits. Bad fittings produced a big leak and required pulling the transmission to get at it before I even started the engine. Then the hydraulic line chafed against the pressure plate, sprung a leak and left me stranded 100 miles from home, my bad for not properly securing it. The Advanced wiring set is substantial in quality but over my head, I wired the front and rear end of the car and engine compartment, but when everything got to the dash I ended up hiring a wiring specialist to complete it. He saved me a lot of headaches. The service from D&D was great, their knowledge of what I needed made a big difference, especially the technical know-how. |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org