| How It Was Done! |  | Your name Jay Fowler |
| --- | --- | --- |
| Conversion performed by Dan Fowler, My Father and myself |  | Town/city and state Amery, Wisconsin |
| Year and model of car 1973 Jensen-Healey MKII |  | E-mail address jfowler@Pressenter.com |
| Engine (make, model, configuration, displacement, etc.) 215 Buick Alum V8 |  | Engine electrical Stock Point GM with MSD |
| Intake (manifold & carburetor, or fuel-injection specifics) Rochester 4-Jet |  | Cooling system Custom Alum Radiator |
| Cam Stock |  | Exhaust system Custom dual exhaust from the manifolds on back |
| Transmission (type, make/model, ratio of top gear, etc.) T-5 from a 1985 Camaro V6 |  | Drive shaft Hybrid ½ GM, ½ Chrysler |
| Clutch System GM Stock |  | Rear axle (make/model, ratio, limited-slip?) Stock Chrysler |
| Front suspension Stock Vauxhall w/HD sway bar |  | Brakes Stock Vauxhall |
| Rear suspension Stock Vauxhall w/HD sway bar |  | Wheels/Tires Stock Wheels, Cooper tires |
| Weight 1975 Lbs dry, no top |  | Electrical (gauges, lighting, audio, etc.) All Stock |
| Miles driven since completion About 12,000 |  | Date conversion completed 1985 |
| Other chassis modifications None |  |  |
| Body and interior modifications Added hardtop 2006, added front scoop for cooling problems. |  |  |
| Performance data 205 Bhp, top speed 130 MPH So Far…. |  |  |
| Sources of parts / information Local junkyards and NAPA |  |  |
| Comments / tips / notable problems encountered during or after completion… The alternator had to be moved to the lower left and a bracket made that attached to the fuel pump. A reservoir tank was made for the radiator to handle cooling issues. The transmission housing is only 3 inches off the ground and the air cleaner rubs the hood. The motor has no fan mounted to it and a manual fan was added to the front of the radiator. During High RPM’s the stock electric fuel pump cant keep up with the demand. It has the original paint, soft top and interior. It was my very first car; I bought it when I was 15 in 1977. |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org