| How It Was Done! |  | Your name STEVE SANETT |
| --- | --- | --- |
| Conversion performed by STEVE SANETT |  | Town/city and state CHATSWORTH, CA |
| Year and model of car 1962 DAIMLER SP250 |  | E-mail address SSANETT@PENTALABS.COM |
| Engine ALL ALUMINUM 427 CHEV. SB DART BLOCK AFR 225CC HEADS, TI VALVES. J.E. PISTONS 12.5:1 CARRILLO RODS, COLA CRANK JESSEL BELT DRIVE |  | Engine electrical MALLORY SUPER MAG. 80 AMP |
| Intake (manifold & carburetor, or fuel-injection specifics) MacKay MAGNESIUM. 48IDA WEBER CARBS BORED TO 51 MM. CUSTOM 46 MM SECONDARY VENTURIS |  | Cooling system RON DAVIS CUSTOM ALUMINUM ALL -24 AERO-QUIP HOSE AND HOSE ENDS |
| Cam SOLID ROLLER BY COMP CAMS 252 DEGREES INT., 248 EX.@.050 .610 VALVE LIFT 108 LOBE SEPARATION |  | Exhaust system CUSTOM 1.750” PRIMARY IN TO OUT SIDE COLLECTORS IN POLISHED STAINLESS STEEL 3.500 SIDE PIPES |
| Transmission (type, make/model, ratio of top gear, etc.) 4 SPEED SUPER T10 CLOSE RATIO TOP GEAR 1:1 |  | Driveshaft CUSTOM |
| Clutch System TILTON TRIPLE ORGANIC DISC . ALUMINUM FLYWHEEL, HYDRAULICALLY ACTUATED. |  | Rear axle (make/model, ratio, limited-slip?) SPEED WAY ENGINEERING QUICK CHANGE WITH DETROIT LOCKER 3.11: 1 |
| Front suspension DAIMLER DOUBLE WISHBONE CONVERTED TO HEIM JOINTS, COIL OVER DBL ADJ ALUM. KONI. CUSTOM SWAY BAR |  | Brakes GIRLING ALUMINUM 18-4, 4 PISTON 11.5 X 1.250 VENTED ROTORS. TILTON PEDAL ASSY. WITH BALANCE BAR |
| Rear suspension LEAF /3 LINK WITH SHOCK TOWERS FOR CONVENTIONAL SHOCKS. KONI DOUBLE ADJ. |  | Wheels/Tires PS ENGINEERING COBRA 8.5 X 15 FRONT 10.5 X 15 REAR. GOOD YEAR BLUE STREAK RACING TIRES |
| Weight 2400 LBS |  | Electrical (gauges, lighting, audio, etc.) ALL TEFLON SILVER WIRE AL MIL STD CONNECTORS ALL SWITCH ARE SILVER CONTACT MIL STD. AIRCRAFT BREAKERS |
| Miles driven since completion 1020 |  | Date conversion completed 2000 |
| Other chassis modifications WELDED ALL FACTORY TUBING. REAR SECTION OF THE FRAME WAS MODIFIED FOR STIFFNESS AND FOR THE SHOCK MODIFICATIONS. THE TOP SHOCK MOUNTING ARE ACTUALLY MOUNTED TO THE ROLL BAR IN THE TRUNK SECTION. SURPRISINGLY WE WERE ABLE TO REUSE THE ORIGINAL ENGINE MOUNTS THAT WERE WELDED TO THE FRAME FOR THE V8 DAIMLER MOTOR. |  |  |
| Body and interior modifications ALL QUARTER PANELS WERE PULLED REARS W ERE VENTED. ALL HANDLES SHAVED. S.S. TRUNK AND HOOD HINGES WERE CUSTOM MADE COBRA REVERSE HOOD SCOOP. BLINKERS ON FRONT REMOVED. FRONT OF THE CAR GRILL RE-SHAPED. SPARCO KEVLAR SEATS 5 POINT SPARCO HARNESS DOOR PANELS & DASH ARE CONNELY HIDE. |  |  |
| Performance data ENGINE DYNO SPECS ARE 540 HP @ 7100 RPM , 575 LB FT TORQUE @ 4150 RPM FRONT TO REAR IS 52% FRONT 48% REAR. ENGINE WAS MOVED BACK 6 INCHES DURING INSTALL. 0-60 IN 3.3 SEC. ¼ MILE ??? NEVER RUN . ESTIMATED MID 10’S ?? |  |  |
| Sources of parts / information MOST OF THE PARTS ARE CUSTOM MADE. SERGE TANK, BREATHER TANK , GAS CAP, BATTERY TRAYS, DASH ARE ALL BILLET INCLUDING THE VALVE COVERS WHICH HAVE LEXAN CLEAR INSERTS BACK ENGRAVED WITH THE DAIMLER LOGO. ON BOARD FIRE EXTINGUISHER SYSTEM. STEWART WARNER MECHANICAL GUAGES. JONES MOTROLA MECHANICAL TACH. DUAL SLA BATTERIES ATL FUEL CELL WITH CUSTOM ALUMINUM ENCLOSURE. ALL BOLTS ARE 12 POINT MS21230 MIL SPEC. I AM SURE THAT I HAVE FORGOTTEN SOME OF THE MODS THAT WE DID. |  |  |
| Comments AVIAID 4 STAGE DRY SUMP OILING SYSTEM. CONVERTED TO RACK AND PINION STEERING. THIS WAS A 10 YEAR PROJECT AND A SIGNIFICANT LEARNING EXPERIENCE AND EXPERIMENT. IF DAIMLER WANTED TO GO RACING AGAINST SHELBY IN THE 60’S , THEN MAYBE THIS WOULD BE AN EXAMPLE OF WHAT WAS POSSIBLE ? WE TOUCHED ON ALL PHASES OF AUTOMOTIVE AND AEROSPACE TECHNOLOGY AND MERGED THEM INTO AN OLD BRITISH CAR. . IT WAS FUN, TEDIOUS AND VERY EXPENSIVE. BUT WORTH EVERY PENNY AND EVERY MINUTE !!! IF IT WAS EXPRESSED IN HOURS ABOUT 5000. IF IT WAS EXPRESSED IN DOLLARS, I NEVER COUNTED !!! |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org