| How It Was Done! | BritishV8 Magazine |  | Car owner’s name- Tim Hoffman |
| --- | --- | --- | --- |
|  | BritishRaceCar.com |  |  |
| Year and model of car- 1979 Jaguar XJ6L |  |  | Town/city and state- Orange, CA |
| Who built or modified this car? Tim Hoffman |  |  | E-mail address and phone- tim79jag383@yahoo.com 562-656-7816 |
| Engine (make, model, configuration, displacement, etc.)- Chevrolet 383 stroker V8. Balanced. 9.5 – 1 compression. |  |  | Engine electrical (including ignition) HEI with MSD6AL |
| World Products S/R Torquer cylinder heads, 2.02, 1.60 valves, Z28 springs. Edelbrock Performer EGR Intake. 600 cfm Edelbrock Performer carb. |  |  | Camshaft and valvetrain- Comp’ Cam #12-235-2 .447/462 lift 210/218 duration 111 separation |
| Cooling and lubrication systems- 4 row GM radiator. Perma-Cool oil cooler with 180 thermostat. |  |  | Exhaust system- Sanderson CC2 headers with air injection added. 2 ½” exhaust, mandrel bends, Magnaflow mufflers. |
| Transmission (type, make/model, ratio of top gear, etc.)- High performance 700r4 from Phoenix Transmissions. |  |  | Clutch System / driveshaft- One piece drive shaft from Inland Empire Driveline |
| Rear axle (make/model, ratio, limited-slip?)- 3.54 |  |  | Brakes (master, front, and rear)- Jaguar ventilated discs front, solid discs rear. |
| Front suspension / steering- Lowered 2” front, stock rear. |  |  | Rear suspension- Stock Jaguar independent, all new bearings, bushings, u-joints. |
| Wheels/Tires- ’88 Jaguar XJ6 wheels, 15 x 7. BFG T/A tires, front 215/70-15. Rear tires 225/70-15. |  |  | Weight- Stock 4200 lbs, with Chevy engine approximately 4000 lbs. |
| Instruments- Stock Jaguar plus Auto Meter oil temperature and Edelbrock Air/Fuel Ratio Monitor. |  |  | Electrical (battery, charging, lighting, audio, etc.) |
| Date completed / Miles (or races) driven since completion |  |  | (racecars only) What class does this car compete in? |
| Other chassis modifications |  |  |  |
| Other body and interior modifications- Louvered hood. Outer headlights converted to ram-air intakes. |  |  |  |
| Performance data |  |  |  |
| Other important info (unusual features / historical notes / comments / tips / problems encountered during or after completion) |  |  |  |

Note: Please send us as many photos as you can.  Additional info is welcome!                         Questions? Call Curtis at 720.652.0237