Engineering Data Summary

Kinematics

Front view virtual swing arm length, 	Front, 62.4 in.		Rear, 59.6 in.

Roll center height, 			Front, 3.0 in.		Rear, 5.0

Scrub radius,				Front, 0.07 in.

Anti-dive, front	13.6%

Dynamics

Bounce mode natural frequency at wheel center,	Front = 1.12 Hz		Rear = 1.05 Hz

Chassis Gradients, estimated

Roll - 3 degree/g,   Dive - 2.6 degree/g (braking),   Squat - 2.8 degree/g (acceleration.)

References

1. Dixon, John, C., Tires, Suspension and Handling, Second Edition, SAE, 1996

2. Dixon, John, C., The Shock Absorber Handbook, SAE, 1999

3. Gillespie, Thomas, D., Fundamentals of Vehicle Dynamics, SAE, 1994

4. Campbell, Colin, New Directions in Suspension Design, Making the Fast Car Faster, R.       	Bentley, 1981

5. Costin, M., Phipps, D., Racing and Sports Car Design, R. Bentley, 1966

6. Smith, S., Advanced Race Car Suspension Development, S. Smith Autosports, 1974

7. Adams, H., Chassis Engineering, HP Books, 1993

8. Milliken, W. F., Milliken, D. L., Race Car Vehicle Dynamics, SAE International, 1995

9. Bastow, D., Howard, G., Car Suspension and Handling, 3rd Edition, SAE, 1993

10. Puhn, F., How to Make Your Car Handle, HP Books, 1981

11. Pacejka, H. B., Tire and Vehicle Dynamics, SAE, 2002

12. Smith, C., Racing Chassis and Suspension Design, PT-90, SAE, 2004

13. Aird, F., Race Car Chassis, Design and Construction, Motorbooks International, 1997

14. Hammill, D., How to Build & Modify Sportscar & Kit Car Suspension & Brakes for Road &                 	Track, Veloce Publishing PLC, 1999

15. Alexander, D., Performance Handling, Motorbooks International, 1991

16. Van Valkenburgh, P., Race Car Engineering & Mechanics, Dodd, Mead & Co., 1976

17. Milliken, W. F., Milliken, D. L., Chassis Design Principles and Analysis, (from notes by M.                	Olley), SAE, 2002

18. Staniforth, A., Race and Rally Car Source Book, The Guide to Building or Modifying a                       	Competition Car, 4th edition, Haynes, 1997

19. Mitchell, W. C., Salinas, J. J., Rowley, W. J., An Introduction to Race Car Engineering, Book 	1, 3rd Edition, Rowley Race Dynamics, 2003